HOME glossary

The HOME vocabulary is at the bottom of the search order.

Home Lexicon:

!			( addr - x )

Stores a 32-bit value.

#ELSE		( -- )

Skips source until #THEN is encountered.

#IF			( f -- )

If f=0, skips source until #THEN or #ELSE is encountered.

#THEN		( -- )

Does nothing.

$DIS			( t-addr -- )

Starts disassembly at an arbitrary address.  Disassembly will proceed until the address of a known word is reached or the ESC key is pressed.

$HLOAD		( $filename -- )

Loads an Intel HEX file into the ROM image.  $filename is a counted string.

$XLOAD-BL51	( $filename -- )

Loads external C labels generated by Keil's 8051 C linker.  $filename is a counted string.

(			( -- )

Skip source until ) is encountered.

((

Skip source until )) is encountered.

(SEE)    		( <name> -- )

Decompiles a host Forth word.

(WORDS)		( -- )

Lists words at the very top of the search order.  For example: when in BUILDING mode, you can use (WORDS) to list the builder lexicon.

*			( x1 x2 -- x3 )

Multiply.  x3 = x1 * x2.

*/			( x1 x2 x3 -- x4 )

Multiply and divide.  x4 = x1 * x2 / x3.

+			( x1 x2 -- x3 )

Add.  x3 = x1 + x2.

-			( x1 x2 -- x3 )

Subtract.  x3 = x1 - x2.

.			( x -- )

Displays signed number from top of stack.

.(			( -- )

Displays source until ) is encountered.

.ASMLABELS	( -- )

Lists all assembler labels.

.FILES		( -- )

Displays all files included in a build.

.HERE		( -- )

Displays the value of the target's code pointer.

.HOT			( -- )

Lists the editor's "hot" keys.

.PROCS		( -- )

Lists external procedures created by somebody else's assembler or C compiler.

.S			( -- )

Displays numbers on the stack.

.SCRAM		( -- )

Displays the ROM scramble table.  See SCRAMBLE.

.STATUS		( -- )

Displays ROM image information.

.VOCS		( -- )

Lists all created vocabularies.

/			( n1 n2 -- quotient )

Signed, floored divide n1 by n2.

//			( -- )

Skips source until end of line.  Like the C version.

/MOD			( n1 n2 -- remainder )

Signed, floored divide n1 by n2.  Returns remainder.

0<			( x -- f )

Returns T if x < 0.

0=			( x -- f )

Returns T if x = 0.

1+			( x -- x+1 )

Adds 1 to top of stack.

1-			( x -- x-1 )

Subtracts 1 from top of stack.

2*			( x -- x*2 )

Multiplies top of stack by 2.

2/			( x -- x/2 )

Divides top of stack by 2 (signed).

<			( x1 x2 -- f )

<=			( x1 x2 -- f )

Returns T if x1 <=  x2.

<>			( x1 x2 -- f )

Returns T if x1 <>  x2.

=			( x1 x2 -- f )

Returns T if x1 =  x2.

>			( x1 x2 -- f )

Returns T if x1 >  x2.

>=			( x1 x2 -- f )

Returns T if x1 >=  x2.

>@@0		( x -- )

>@@1		( x -- )

>@@2		( x -- )

>@@3		( x -- )

>@@4		( x -- )

>@@5		( x -- )

>@@6		( x -- )

>@@7		( x -- )

>@@8		( x -- )

>@@9		( x -- )

Store to local assembler label.

>ADDRNIBBLES	( x -- )

Set the target's debugger address width in nibbles.

>CELLBITS		( x -- )

Set the target's bits per cell.

>CHARBITS		( x -- )

Set the target's bits per character.

>FLAGS		( x -- )

Store to the flags field of the last created header.

>LITERAL		( x -- )

Store to the literal field of the last created header.

>PORT#		( x -- )

Set the parallel port# for a ROM emulator.

>SHOWME		( f -- )

Stores a flag to a variable that's sometimes used by the compiler to show what's going on.  Mostly for debugging compilers.

>TOKEN#		( n -- )

Store to TOKEN#.  Same as {{ TO TOKEN# }}.

@			( addr -- x )

Fetch 32-bit value from address.

ADD-TOKEN    	( xt spos sid dt cfa <name> -- )

Adds a token header to the current vocabulary.

xt = token#.

spos, sid = file position and source file id.

dt = datatype     cfa = image address.

ADDFILE		( id# <filename> -- )

Add a filename to the file ID list.  See NOFILES.

ADDRNIBBLES	( -- x )

Get the target's debugger address width in nibbles.

ADDWATCH   	( <name> -- )

Adds a word to the watch list.

ALIGNMENT   	( -- x )

Gets the larger alignment value of code and data space.

ALSO     		( -- )

Duplicates the top of the search order.

AND			( x1 x2 -- x3 )

Logical AND.  x3 = x1 & x2.

ANEW		( <name> -- )

If <name> exists, execute it.  Otherwise, compile a word that removes everything after itself in the dictionary.

ASMARRAY		( #bytes <name> -- )

Defines an assembler label and allots space for it.  Use HERE and ORG to set start addresses, etc.

ASMBYTE		( <name> -- )

Defines an assembler label and allots one byte for it.  Use HERE and ORG to set start addresses, etc.

ASMLABEL		( x <name> -- )

Defines an assembler label, value is x.

ASMLABEL?   	( <name> -- x )

Look up an existing assembler label.

ASMLONG		( <name> -- )

Defines an assembler label and allots four bytes for it.

ASMWORD		( <name> -- )

Defines an assembler label and allots two bytes for it.

B			( -- )

Browses the current file.

BASE			( -- addr )

Address of numeric conversion radix.

BETWEEN		( n1 n2 n3 -- f )

Signed compare.  Returns T if n1 <=  n3 <= n2.

BI			( -- )

Enters the ROM builder mode.

BINARY		( -- )

Sets BASE = 2.

BOUNDS>NA	( addr-hi addr-lo -- #cells addr )

Converts address bounds to base address and cell count.

BRANCHBYTES	( -- x )

Gets # of bytes of a control structure branch, set by the target-specific builder file.

BROWSE		( <filename> -- )

Browses a file using Winview.

BSAVE		( <filename> -- )

Saves the ROM image (up to HERE) in binary format.

BUG			( <name> -- )

Invokes the high level debugger.  Opens the source file to the start position and makes sure target is in test mode.

BUILDING		( -- )

Enters the ROM builder mode.

BYE			( -- )

Exits to Windows.  Same as Alt-F4.

BYTE-JOIN		( lo hi -- x )

Joins two bytes to form a 16-bit value.

BYTE-SPLIT		( x -- lo hi )

Splits a 16-bit value to form two bytes.

C(			( string -- )

Store a comment string, up to 32 characters long and delimited by ')', to the comment field of the last created header.

CALIGNMENT	( -- n )

Alignment value for code space.

CELL     		( -- n )

Cell size in address units on the target.

CELLBITS		( -- n )

Target bits per cell.

CELLS		( n1 -- n2 )

Multiplies by the target's cell address units.

CHARBITS		( -- n )

Target bits per character.

CHARS		( n1 -- n2 )

Multiplies by the target's character address units.

CHECKSUM		( addr len -- checksum )

Computes the checksum of a block of image data.

CLEARWATCH	( -- )

Clear the watch list.

CLS			( -- )

Clear the console.

CODE-BOUNDS	( addr-lo addr-hi -- )

Set boundaries for code space.

CODE-BOUNDS?	( -- addr-lo addr-hi )

Get boundaries for code space.

CODETEMPLATE	( <filename> -- )

Creates a listing that can be used as a template for somebody else's assembler for porting to a new processor.  Not very useful, but you can modify this word to generate other formats.

CONSTANT		( x <name> -- )

Creates a constant.

CPUTYPE		( -- x )

Returns the CPU type being compiled for.

CR			( -- )

Starts a new line on the console.

CREATE		( <name> -- )

Creates a data structure.

DALIGNMENT	( -- n )

Alignment value for data space.

DATA-BOUNDS	( addr-lo addr-hi -- )

Sets boundaries for data space.

DATA-BOUNDS?	( addr-lo addr-hi -- )

Gets boundaries for data space.

DATATYPE		( x -- )

Stores to the datatype field of the last created header.

DECIMAL		( -- )

Sets BASE = 10.

DEFINITIONS	( -- )

Makes the top vocabulary of the search order the current vocabulary.

DROP			( x -- )

Discards top stack item.

DUP			( x -- x x )

Duplicates top stack item.

DYNAMIC		( -- )

Sets compiler to use binding table.

E			( -- )

Edits the current file.

EDIT			( <filename> -- )

Edits a file using Winview.

FI			( -- )

Enters the interactive Forth mode.

FLOAD		( <filename> -- )

Loads a Forth source file.

FORTH		( -- )

Places the FORTH wordlist at the top of the search order.

FORTHING		( -- )

Enters the interactive Forth mode.

FSEND		( <filename> -- )

Send a bytecode file to the target and evaluate it.

HEX			( -- )

Sets BASE = 16.

HEX[			( -- )

Lays down incoming blank-delimited hex numbers until an unrecognizable number or end of line is encountered. Usage: hex[ 0AB 07F ... 0FC ]

HLOAD		( <filename> -- )

Loads an Intel HEX file into the ROM image.

HOME		( -- )

Places the HOME wordlist at the top of the search order.

HOMEORDER	( -- )

Makes the HOME wordlist the only one in the search order and directs host definitions to the HOME wordlist.

HOST			( -- )

Enters the host's Forth mode.

HSAVE		( <filename> -- )

Saves the ROM image (up to HERE) in Intel HEX format.

INCLUDE		( <filename> -- )

Includes a Forth source file.  Same as FLOAD.

INCLUDE-BINARY	( <filename> -- )

Appends raw binary data to the image.

INVERT		( x1 -- x2 )

Bitwise-inverts x1.  Same as -1 XOR.

IP?			( -- )

Displays the current IP address of the remote server.

IP=			( <string> -- )

Sets the current IP address of the remote server.

IP:			( -- )

Fetches the current IP address of the remote server.

LOADLABELS	( -- )

Uploads addresses from the target's binding table to enable low level debugging.

LOADROM		( -- )

Blasts image data out the parallel port to a ROM emulator.

LOW-TOKENS	( -- )

Selects the low set of token values.

LSHIFT		( n1 n2 -- n3 )

Logical left shift.  n3 = n1 << n2.

MAIN-TOKENS	( -- )

Selects the main set of token values.

MAKEDEFINING	( <name> -- )

Places a new defining word in the TOKENIZER wordlist.

MARKER		( <name> -- )

Compiles a word that removes everything after itself in the dictionary.

MAXUINT		( -- x )

Maximum unsigned integer available on target.

NEEDS		( <filename> -- )

FLOADs a file if it hasn't been loaded yet.

NEGATE		( n -- -n )

Negates n.

NEW-IMAGE   	( -- )

Clears the image and pointers.

NIP			( n1 n2 -- n2 )

Discards second item on stack.

NOFILES		( -- )

Clears the file ID list.

NOSLACK		( -- addr )

Flag:  True if warnings abort compilation.

OFF			( addr -- )

Stores 0 to address.

OK			( -- )

FLOAD the current file.

ON			( addr -- )

Stores -1 to address.

ONLY			( -- )

Clears the search order so it only contains the HOME wordlist.

OR			( x1 x2 -- x3 )

Logical OR.  x3 = x1 | x2.

ORDER		( -- )

Displays the current search order.

ORG			( t-addr -- )

Sets new start address.

OVER			( n1 n2 -- n1 n2 n1 )

Copies second item on stack.

PICK			( ... k -- ... n[k] )

Copies kth item on stack.

PREVIOUS		( -- )

Drops the top vocabulary in the search order.

RESET		( -- )

Drops the target board’s DTR line for ½ second.

ROM-BOUNDS	( addr-lo addr-hi -- )

Sets boundaries for ROM image.

ROM-BOUNDS?	( addr-lo addr-hi -- )

Gets boundaries for ROM image.

ROT			( n1 n2 n3 -- n2 n3 n1 )

Rotate top 3 items on stack.

RSHIFT		( n1 n2 -- n3 )

Logical right shift.  n3 = n1 >> n2.

SCRAM-A		( cpu-pin rom-pin -- )

Re-map an address bus pin.

SCRAM-D		( cpu-pin rom-pin -- )

Re-map a data bus pin.

SCRAMBLE		( -- )

Scramble ROM image for byte-wide ROM

SCRAMBLE-INIT	( -- )

Clear the ROM scramble table.

SCRAMBLE16	( -- )

Scramble ROM image for 16-bit-wide ROM

SCRAMBLE32	( -- )

Scramble ROM image for 32-bit-wide ROM

SEE			( <name> -- )

Disassembles <name>.  If <name> is tokenized code, the bytecode viewer is used.  Otherwise, the disassembler is used.  Hit ESC to quit early.

SF			( <fracnumber> -- n )

Gets a signed fraction between -1.0 and +1.0 from the input stream and converts it to integer for the target's cell width.  Example: on a 16-bit target, SF +0.5 maps to 16384.

SHELL		( -- )

Opens a DOS window.

SSAVE		( <filename> -- )

Saves the ROM image (up to HERE) in Motorola S format.

STATIC		( -- )

Sets compiler to ignore (bypass) binding table.

SWAP  		( n1 n2 -- n2 n1 )

Swap the top two stack items.

TAR?			( -- )

Display rather comprehensive target information.

TE			( -- )

Enters testing mode.  For interactive testing of target words.

TEMP-TOKENS	( -- )

Selects the temporary set of token values.

TESTING		( -- )

Enters testing mode.  For interactive testing of target words.

TI			( -- )

Enters tokenizing mode.

TO			( x <name> -- )

Stores to a value.

TOKEN#		( x -- )

The next free token#.

TOKENIZING	( -- )

Enters tokenizing mode.

TRACE		( <name> -- )

Invokes the low level debugger.  Opens the debugger window and displays the disassembled code.

TSAVE		( <filename> -- )

Saves the token list as an FLOADable file.

TUCK   		( n1 n2 -- n2 n1 n2 )

Tuck the first item under the second.

U2/			( n1 -- n2 )

Unsigned divide by 2.  Same as 1 RSHIFT.

UF			( <fracnumber> -- n )

Gets a signed fraction between 0 and +1.0 from the input stream and converts it to integer for the target's cell width.  Example: on a 16-bit target, UF 0.5 maps to 32768.

UF1.0    		( -- d )

Returns double cell value: 2^cellsize.

UM*			( u1 u2 -- ud )

Unsigned multiply, result is a double.

UM/MOD		( ud u -- remainder quotient )

Unsigned divide: double / single.

UNDERALSO	( -- )

Duplicates the second wordlist in the search order.

UNDERDEFS	( -- )

Makes the second wordlist of the search order the current wordlist.

UNDERPREVIOUS	( -- )

Removes the second wordlist in the search order.

VALUE		( x <name> -- )

Creates a 32-bit value.  Acts like CONSTANT except that you can use TO VALUE to change it.

VARIABLE		( <name> -- )

Creates a 32-bit variable.

VIEW     		( <name> -- )

View/browse the source code of <name>.

VMTEMPLATE	( <filename> -- )

Creates a listing that can be used as a template for somebody else's Forth for implementing a new VM.  Not very useful, but you can modify it to generate other formats.

VOCABULARY	( <name> -- )

Creates a word that replaces the top of the search order with its own vocabulary.

WARNING		( -- addr )

Flag: True if redefinitions cause a warning message.

WATCHCODE"	( address format string" -- )

Adds a memory watch record to the watch list.  Address is the target memory address to watch, format is how it's displayed, and string" is a quote-delimited description.  The format number is two bytes: the upper byte is the display format and the lower byte is the byte count.  Formats are: 0=byte, 1=16bit-LE, 2=16bit-BE, 3=32bit-LE, 4=32bit-BE, 5=ASCII.

WATCHDATA"	( address format string" -- )

Adds a data memory watch record.

Example: 0xC000 0x102 WATCHDATA" Location C000 = "

adds an item to the watch list that fetches 2 bytes from data memory location 0xC000 and displays the data as a 16-bit little-endian number.

WATCHREG"	( address format string" -- )

Adds a register watch record.

WATCHEE"		( address format string" -- )

Adds a non-volatile memory watch record.

WITHIN		( u1 u2 u3 -- f )

Unsigned compare.  Returns T if u1 <=  u3 < u2.

WORD-JOIN		( lo hi -- x )

Joins two 16-bit values to form a 16-bit value.

WORD-SPLIT   	( x -- lo hi )

Splits a 16-bit value to form two 16-bit values.

WORDS		( -- )

Displays all words in the vocabulary at the top of the search order.

XLOAD-BL51	( <filename> -- )

Loads external C labels generated by Keil's 8051 C linker.

XOR			( x1 x2 -- x3 )

Logical exclusive OR.  x3 = x1 ^ x2.

[ELSE]   		( -- )

Skips source until [THEN] is encountered.

[IF]			( f -- )

If f=0, skips source until [THEN] or [ELSE] is encountered.

[THEN]		( -- )

Does nothing.

{{			( ? -- ? )

Interpret the input stream until }}.  Uses only HOME in the search order and assumes there is no compilation.

\			( -- )

Skip source until end of line.

\S						( -- )

Skip the rest of the file.

}}			( -- )

Ends a {{ ... }} phrase.

~BOOT0		( -- )

Downloads the tokenized program in the image to the target board's primary boot area.

~BOOT1		( -- )

Downloads the tokenized program in the image to the target board's primary boot area.

_DBG     		( <name> -- )

Starts Win32forth's debugger, includes Forth at the top of the search order.  To use this, you must set OldStartup to 1 and recompile the Firmware Studio source.  This is good for debugging Firmware Studio itself.

_DEBUG		( <name> -- )

Similar to _DBG but doesn't start debugger until <name> is executed.